BOSTON

INTERNATIONAL COLLEGE

END TERM EXAM

| Level: Bachelor |  | Year : 2024 |
| --- | --- | --- |
| Program: BCSIT Sem: II |  | Full Marks : 100 |
| Course: Digital Systems |  | Time : 3 hrs |

Candidates are required to give their answers in their own words as far as practicable. The figures in the margin indicate full marks.

# Group A

Very Short Answer Questions

Attempt all questions. Give your answer in few lines.                10*2=20

- Differentiate between Analog and Digital systems.

- Mention the steps to get the dual of any Boolean expression with an example.

- Name two universal gates. Why are they called universal gates?

- Mention methods of triggering flipflops.

- Find the base X if: (165)8= (313) x.

- How many selection lines are required for a multiplexer of 127*1?

- Define one bit memory.

- How many flipflops are required to design the MOD 99 counter?

- Define Flags. Mention its types.

- Define CMOS and mention its application.

# Group B

Short Answer Questions

Attempt any five questions.					     5*10=50

- Perform the following conversion

a.	(34D.5A)16=(?)10

b.	(100110101) gray=(?)2

c.	(344)10=(?)8

d.   (1156)8=(?)16

- Perform (545-57)10 using 9’s and 10’s complement.

- Design full adder using 2 half adder and OR gate.

- What do you mean by a counter? Design a 3-bit synchronous counter.

- Explain in detail the operation of JK flip-flops along with the truth table, and characteristics equation.

- Explain the operation of the Parallel in Parallel Out (PIPO) shift register along with the timing diagram.

Group C

Long Questions	(3*10=30)

- Simplify the following expression.

F=m(0,2,5,9,15)+d(1,4,7,12)

Using k-map and draw the logic diagram using

- Basic gates only

- NAND gates only

- Difference between multiplexer and demultiplexer. Design 8*1 mux using 4*1 mux.

- Design a sequential circuit from the following state diagram using the flipflop of your choice.